Vital signs:

The body temperature , the pulse or rate of heartbeats, the respiration or rate of breathing and the blood pressure.

The vital signs are abbreviated :

TPR

For :temperature , pulse respirations

BP

For: blood pressure.

These readings are called vital signs ?

Because they all must be present for life to continue.

Temperature :

1 - Being human , we are homeothermic; we are warm-blooded and maintain body temperature independently of:

1- our environment .

2- our body generates heat as it burns food .

It loses heat through:

1- the lungs (breathing )

2 -through the skin (sweating ),

3- in body discharges ( urine , feces, vomitus ,or blood).

Body temperature is defined as:

1 -The measure of the heat inside the body

2 -The balance between heat produced and heat lost .

Types of thermometers:

1 – clinical

2- Electronic

3 – disposable

4 – patch

Clinical:

The clinical thermometer is a glass bulb containing mercury , with stem in which the mercury can rise .

The stem has lines representing the measuring scale.

1 - It must read below normal range before the temperature is taken .

2 - it should be rinsed in cold water to avoid distribution of the mercury and  breakage .

The clinical thermometer may be oral or rectal .

| used |  |  |
| --- | --- | --- |
| axillary mouth | Long slender bulb | Oral thermometer |
| rectal | Blunt Short Fat bulb | Rectal thermometer |

The rectal thermometer it should not be stored with

The oral thermometers.

: Electronic

1 – is portable and battery operated .

2 – it registers the temperature in 10 seconds or less and displays it digitally.

3- it must be fully charged to give an accurate reading .

4 – separate oral and rectal probes are supplied with each unit .

Disposable:

1 – single – use

2 – has a sensor at the end of the shaft which measures temperature.

Patch:

1 – a strip, which contains liquid crystals that change colors as temperature changes .

2 – it is usually placed on the forehead.

3- the scale is adjusted to convert skin-surface temperature to inner- body temperature .

4 – the calibration is not as detailed as that of glass thermometer.

Sites for taking the body temperature :

A thermometer is placed in the patient's :

1 – mouth to obtain an oral temperature

2- in the anal canal to obtain a rectal temperature

3 – in axilla (armpit) to obtain an axillary temperature.

| way | time |  |
| --- | --- | --- |
| 1 – place the thermometer in the sublingual pocket and have the patient close his mouth around it. 2 – instruct him not to bite down. 3 –if the patient has been eating , drinking , smoking , brushing has teeth, or chewing gum within the past 15 minutes , wait at least 15 minutes to take the temperature. | For 2-3 Minutes | Oral temperature |
| 1 –lubricate the bulb and the area up to 1 inch above it. 2 –use a lubricate probe cover with an electronic thermometer. 3- turn the patient on his a side, fold back the bedding and separate the buttocks so that you can easily see the anal opening. 4 – insert the thermometer approximately 1.5 inches into the anus . | For 2 minutes | Rectal temperature |
| 1 – place the thermometer in a dry axilla . 2 - Keep the arm close to the body to ensure contact with the bulb. 3 – axillary is the method of choice for an infant. | For 7-10 minutes | Axillary temperature |

Precautions :

1 – oral temperatures are contraindicated for:

A – an unconscious patient

B – infant

C – when the patient must breathe  through the mouth

2 – the rectal method of obtaining the temperature is contraindicated if :

A – the patient has diarrhea

B – rectal disease

C – has recently had rectal surgery .

Body temperature :

1 – body temperature rises when ( heat production increases + or when heat loss decreases )

Both may be going on at the same time .

2 – when the temperature is elevated , the pyrexia or a fever is present .

3 – a fever is a symptom of some disorder .

4 – it often accompanies illness usually when the body is fighting an infection.

5 – An antipyretic is a fever – reducing agent such as aspirin

6 – a temperatures significantly below normal is called (hypothermia)

7- such temperatures often precede normal death .

8 – hypothermia may occur as result of overexposure to cold.

9 – accidental hypothermia is life threatening and

must be treated immediately .