International Academy for Health Science

Department :Dental Assistant                                                                 Level : 3

Subject: Dental Radiology II

Student Name:………………………………………….                          Period :Evening

………………………………………………………………………………..................................................

Choose The Correct Answer : (30 marks)

1-  Over dose of radiation may cause

  - leukemia

  - skin changes

  - cataracts

  - all of the above

2-............is the measurement of ionization

.              a- Rad

- Rem

- Roentgen

d-All of the above

3- Radiation causes cell death by

a- Changing in nucleoproteins

b- Destroying them into chondria

c- Ionization

d- None-of the above

4- Filter are used in radiography to

a- Reduce patient radiation

b- Reduce short wavelength rays

c- Increase secondary radiation

d- Increase image contrast

5- Size of the X-ray beam may be reduced by :

a- Aluminum filter

b- Tungsten tube

c- Lead diaphragm

d- All of the above

6- A dark film indicated that it has been :

a- Overdeveloped

b- Underdeveloped

c- Exposed to light

d- Overfixed

7- In order to reduce the exposure time the factor to be increased is

a- Kilovoltage

b- Milliamperage

c- Exposure time

d- Distance

8- ....................is preferred because less divergence of x-ray beam occurs

a- Long cone

b- Short cone

9- Thyroid collars are recommended with

a- Extra-oral films

b- Intra- oral films

10- The distance between the source of x-ray and the operator should be at least

a- 3 feet

b- 6 feet

c- 10 feet

d- 8 feet

11- Unnecessary radiation in the dental office may be due to

a- Routine full mouth exposure

b- Poor technique of exposure

c- Poor technique of processing

d- All of the above

12- Who should held a film in place in the patient's mouth during exposure?

a- Dental assistant

b- Operator

c- Patient

d- None- of the above

13- ..................is a device that measures the irradiation dose accumulated by individuals over any given period of time.

a- Dosimeters

b- Shields

c- A direct switch

d- All of the above.

14- The following are disadvantages of the paralleling technique except

a- Radiation exposure to patient fingers

b- Need special equipments

c- The technique is difficult in small mouth

d- Sometimes the apical area not appears completely

15- Bite- wing radiograph is useful in diagnosing of

a- overhanging restoration

b- Periapical bony lesions

c- Impaction

d- Un-erupted teeth

16- The disadvantage of the film holders is

a- Difficulty in checking film position

b- Gagging sensation

c- Patient discomfort

d- All of the above

17- The vertical angulation (cone position) to radiograph lower premolar should be

a- zero

b- 10

c- 20-25

d- 15-20

18- The vertical angulation is decreased by 5 in case of

a- High arched palate

b- low palatal vault

c- Flat ridges

d- Labially inclined teeth

19- In case of trismus as in impacted 3rd tooth where the patient can not open his mouth the best radiographic tenhnique is the

a- Periapical radiograph

b- Lateral oblique view

c- Bit-wing radiograph

d- Occlusal view

20- Elongation or shortening of the image can be controlled by

a- Altering the vertical angulation

b- Altering the horizontal angulation

c- Altering the source-film distance

d- None of the above

21- Best X-ray view for displacement of the condyles is

a- Panoramic

b- Water's

c- Revers Town's

d- P-A view

22- Maxillary sinus is best viewed on

a- P-A view

b- Towne's view

c- Water's view

d- Submentovertex view

23- CT scan is useful in

a- Dentofacial fracture&lesion

b- Assessment of jaw bone for implant

c- TMJ disorders

d- All of the above

24- The radiographic view taken with the film and x-ray tube are rotated simultaneously around a patient during exposure is

a- Occlusal view

b- Panoramic view

c- Water's

d- PNS view

25- Fractures of the outer or inner plates of the frontal sinus are best viewed on

a- Dead lateral view

b- Towne's view

c- Submentovertex view

d- None-of the above

26- Overexposure result in

a- Fogging

b- Light film

c- Dark film

d- Less contrast

27- Blurred image is due to

a- High temperature

b- Increasing KVP

c- Overexposure

d-  Patient Movement during exposure

28- Yellowish brown discoloration in X-ray is due to

a- Low temperature

b- High temperature

c- Insufficient washing

d- Exposure to light

29- Fogging on the radiograph is caused by

a- Increase in humidity

b- Improper safe lighting

c- Overdevelopment

d- All of the above

30- Radiographic film may appear too light because of using

a- A concentrated developer solution

b- Overexposure of film

c- Depleted developer solution

d- Incorrect safe light

II-Complete The Following (10 marks)

1-. ..............vertical angulation.....is the angle formed between the central ray and occlusal plane.

2-Using the fast film is of great importance in reducing.exposure time.........................and..patient's dose........................

3-In paralleling technique the x-ray film is supported parallel to...long axis of the tooth........................

- In case of flat ridge the vertical angulation is increased by...5.................

- The radiographic view taken with the patient chin placed on the cassette is....water's view.......................

- ....tomography....................is radiography technique that allows the imaging of one layer or section of the body while blurring images of structures in other planes.

- Patient movement during exposure will result in..blurred ..................image.

- In case of bite-wing technique incorrect vertical angulation leads to.......distorted.................image

- The length of the short cone is ....16............inches

- Excessive fixation will result in......light....................radiograph.

- Say Whether The Following Statments Are True or False(10 marks)

- Slow film preferable more than the fast film.f

- The rectangular cone type is the least effective in reducing patients exposure from the three types.f

- Diamter of X-ray beam must excced 2.75 inches.f

- Using x-ray machines of high Kvp increases the patient's dosage.f

- The operator should stand in contact to the patient during exposure.f

- E-speed film is as twice as fast as the D-speed film.t

- Overexposure will result in dark radiogrph.t

- Distoration is the result of film over bending.t

- Excessive fixation will result in light radiograph.t

- Insufficient washing will result in yellowish brown discoloration.t

IV- Enumrate five uses for bite-wing radiograph ? ( 5 marks)

AZMI